## Knowing Which Card Is Worth It

To make things easy on you, the following chipsets are known to support monitor mode and packet injection per our testing:

- Atheros AR9271: The Alfa AWUS036NHA is my favorite long-range network adapter and the standard by which I judge other long-range adapters. It's stable, fast, and a well-supported b/g/n wireless network adapter. There's also the TP-Link TL-WN722N, a favorite for newbies and experienced hackers alike. It's a compact b/g/n adapter that has one of the cheapest prices but boasts surprisingly impressive performance. That being said, only v1 will work with Kali Linux since v2 uses a different chipset.

- Ralink RT3070: This chipset resides inside a number of popular wireless network adapters. Of those, the Alfa AWUS036NH is a b/g/n adapter with an absurd amount of range. It can be amplified by the omnidirectional antenna and can be paired with a Yagi or Paddle antenna to create a directional array. For a more discreet wireless adapter that can be plugged in via USB, the Alfa AWUS036NEH is a powerful b/g/n adapter that's slim and doesn't require a USB cable to use. It has the added advantage of retaining its swappable antenna. If you need a stealthier option that doesn't look like it could hack anything, you might consider the g/n Panda PAU05. While small, it's a low profile adapter with a strong performance in the short and medium range, a reduced range for when you want to gather network data without including everything within several blocks.

- Ralink RT3572: While the previous adapters have been 2.4 GHz only, the Alfa AWUS051NH v2 is a dual-band adapter that is also compatible with 5 GHz networks. While slightly pricier, the dual-band capacity and compatibility with 802.11n draft 3.0 and 802.11a/b/g wireless standards make this a more advanced option.

- Realtek 8187L (Wireless G adapters): The Alfa AWUS036H USB 2.4 GHz adapters use this older chipset that is less useful and will not pick up as many networks. These cards still will work against some networks, thus are great for beginners, as there are a ton around for cheap.

- Realtek RTL8812AU: Supported in 2017, the Alfa AWUS036ACH is a beast, with dual antennas and 802.11ac and a, b, g, n compatibility with 300 Mbps at 2.4 GHz and 867 Mbps at 5 GHz. It's one of the newest offerings that are compatible with Kali, so if you're looking for the fastest and longest range, this would be an adapter to consider. To use it, you may need to first run "apt update" followed by "apt install realtek-rtl88xxau-dkms" which will install the needed drivers to enable packet injection.

Aircrack-ng also lists a few cards as best in class on its site, so if you're interested in more suggestions, check it out (some of the ones listed above are also on its list). Also, check out our head-to-head test of wireless network adapters compatible with Kali Linux.

## Other Considerations in Adapter Selection

Aside from the chipset, another consideration is the frequency on which the adapter operates. While most Wi-Fi devices, including IoT devices, operate on the older 2.4 GHz band, many newer devices also offer 5 GHz networks. These networks are generally faster and can transfer more data, but are also usually paired with a 2.4 GHz network. The question when purchasing then becomes, is it worth it to invest the extra money in a 2.4/5 GHz antenna that can detect (and attack) both?

In many cases, unless the point of your attack is to probe all of the available networks in an area, a 2.4 GHz card will be fine. If 5 GHz is important to you, there are many 5 GHz Wi-Fi cards that support monitor mode and packet injection, an example being the Panda Wireless Pau09.

Another important factor is determining whether you need to mount a specialized antenna. While most omnidirectional antennas will be fine for a beginner, you may want to switch to an antenna with a directional pattern to focus on a particular network or area rather than everything in a circle around you. If this is the case, look for adapters with antennas that can be removed and swapped with a different type.

## Option 2Test Your Existing Wireless Network Adapter

If you already have a wireless network adapter, you can check pretty easily if the chipset inside supports monitor mode and packet injection. To start, plug in the network adapter and then open a terminal window. You should be able to determine the chipset of the network adapter by simply typing lsusb -vv into the terminal window and looking for an output similar to below.

lsusb -vv

Bus 001 Device 002: ID 148f:5372 Ralink Technology, Corp. RT5372 Wireless Adapter

Device Descriptor:

bLength                18

bDescriptorType         1

bcdUSB               2.00

bDeviceClass            0 (Defined at Interface level)

bDeviceSubClass         0

bDeviceProtocol         0

bMaxPacketSize0        64

idVendor           0x148f Ralink Technology, Corp.

idProduct          0x5372 RT5372 Wireless Adapter

bcdDevice            1.01

iManufacturer           1 Ralink

iProduct                2 802.11 n WLAN

iSerial                 3 (error)

bNumConfigurations      1

In my example, I'm looking at a Panda Wireless PAU06 network adapter, which reports having an RT5372 chipset from Ralink, which is listed as supported! Once you know the chipset of your card, you should have a rough idea of what it can do.

## Testing Your Adapter's Abilities

Now, let's move on to more active testing of the adapter's capabilities.

## Step 1Put Your Card in Monitor Mode

For this step, we'll break out Airmon-ng, but before that, you'll need to locate the name of the interface. On your system, run the command ifconfig (or ip a) to see a list of all devices connected. On Kali Linux, your card should be listed as something like wlan0 or wlan1.

ifconfig

eth0: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

inet 10.0.2.15  netmask 255.255.255.0  broadcast 10.0.2.255

inet6 fe80::a00:27ff:fe59:1b51  prefixlen 64  scopeid 0x20<link>

ether 86:09:15:d2:9e:96  txqueuelen 1000  (Ethernet)

RX packets 700  bytes 925050 (903.3 KiB)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 519  bytes 33297 (32.5 KiB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

lo: flags=73<UP,LOOPBACK,RUNNING>  mtu 65536

inet 127.0.0.1  netmask 255.0.0.0

inet6 ::1  prefixlen 128  scopeid 0x10<host>

loop  txqueuelen 1000  (Local Loopback)

RX packets 20  bytes 1116 (1.0 KiB)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 20  bytes 1116 (1.0 KiB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

wlan0: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

ether EE-A5-3C-37-34-4A  txqueuelen 1000  (Ethernet)

RX packets 0  bytes 0 (0.0 B)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 0  bytes 0 (0.0 B)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

Once you have the name of the network interface, you can attempt to put it into monitor mode by typing airmon-ng start wlan0 (assuming your interface name is wlan0). If you see the output below, then your card appears to support wireless monitor mode.

airmon-ng start wlan0

Found 3 processes that could cause trouble.

If airodump-ng, aireplay-ng or airtun-ng stops working after

a short period of time, you may want to run 'airmon-ng check kill'

PID Name

428 NetworkManager

522 dhclient

718 wpa_supplicant

PHY	Interface	Driver		Chipset

phy1	wlan0		rt2800usb	Ralink Technology, Corp. RT5372

(mac80211 monitor mode vif enabled for [phy1]wlan0 on [phy1]wlan0mon)

(mac80211 station mode vif disabled for [phy1]wlan0)

You can confirm the results by typing iwconfig, and you should see the name of your card has changed to add a "mon" at the end of your card's name. It should also report "Mode:Monitor" if it has been successfully put into monitor mode.

iwconfig

wlan0mon  IEEE 802.11  Mode:Monitor  Frequency:2.457 GHz  Tx-Power=20 dBm

Retry short  long limit:2   RTS thr:off   Fragment thr:off

Power Management:off

## Step 2Test Your Card for Packet Injection

Testing for packet injection is fairly straightforward to test thanks to tools included in Airplay-ng. After putting your card into monitor mode in the last step, you can run a test to see if the wireless network adapter is capable of injecting packets into nearby wireless networks.

Starting with your interface in monitor mode, make sure you are in proximity to a few Wi-Fi networks so that the adapter has a chance of succeeding. Then, in a terminal window, type aireplay-ng --test wlan0mon to start the packet injection test.

- Don't Miss: Disable Cameras on Any Wireless Network with Aireplay-ng

aireplay-ng --test wlan0mon

12:47:05  Waiting for beacon frame (BSSID: AA:BB:CC:DD:EE) on channel 7

12:47:05  Trying broadcast probe requests...

12:47:06  Injection is working!

12:47:07  Found 1 AP

12:47:07  Trying directed probe requests...

12:47:07  AA:BB:CC:DD:EE - channel: 7 - 'Dobis'

12:47:08  Ping (min/avg/max): 0.891ms/15.899ms/32.832ms Power: -21.72

12:47:08  29/30:  96%

If you get a result like above, then congratulations, your network card is successfully injecting packets into nearby networks. If you get a result like the one below, then your card may not support packet injection.

aireplay-ng --test wlan0mon

21:47:18  Waiting for beacon frame (BSSID: AA:BB:CC:DD:EE) on channel 6

21:47:18  Trying broadcast probe requests...

21:47:20  No Answer...

21:47:20  Found 1 AP

21:47:20  Trying directed probe requests...

21:47:20  74:85:2A:97:5B:08 - channel: 6 - 'Dobis'

21:47:26   0/30:   0%

## Step 3Test with an Attack to Make Sure Everything Works

Finally, we can put the above two steps into practice by attempting to capture a WPA handshake using Besside-ng, a versatile and extremely useful tool for WPA cracking, which also happens to be a great way of testing if your card is able to attack a WPA network.

- Don't Miss: Automating Wi-Fi Hacking with Besside-ng

To start, make sure you have a network nearby you have permission to attack. By default, Besside-ng will attack everything in range, and the attack is very noisy. Besside-ng is designed to scan for networks with a device connected, then attack the connection by injecting deauthentication packets, causing the device to momentarily disconnect. When it reconnects, a hacker can use the information exchanged by the devices to attempt to brute-force the password.

Type the besside-ng -R 'Target Network' wlan0mon command, with the -R field replaced with the name of your test network. It will begin attempting to grab a handshake from the victim network. For this to work, there must be a device connected to the Wi-Fi network you're attacking. If there isn't a device present, then there is no one to kick off the network so you can't try to capture the handshake.

besside-ng -R 'Target Network' wlan0mon

[21:08:54] Let's ride

[21:08:54] Resuming from besside.log

[21:08:54] Appending to wpa.cap

[21:08:54] Appending to wep.cap

[21:08:54] Logging to besside.log

If you get an output like below, then congratulations! Your card is capable of grabbing handshakes from WPA/WPA2 networks. You can also check out our guide on Besside-ng to understand more about what a Besside-ng attack is capable of.

besside-ng wlan0mon

[03:20:45] Let's ride

[03:20:45] Resuming from besside.log

[03:20:45] Appending to wpa.cap

[03:20:45] Appending to wep.cap

[03:20:45] Logging to besside.log

[03:20:56] TO-OWN [DirtyLittleBirdyFeet*, Sonos*] OWNED []

[03:21:03] Crappy connection - Sonos unreachable got 0/10 (100% loss) [-74 dbm]

[03:21:07] Got necessary WPA handshake info for DirtyLittleBirdyFeet

[03:21:07] Run aircrack on wpa.cap for WPA key

[03:21:07] Pwned network DirtyLittleBirdyFeet in 0:04 mins:sec

[03:21:07] TO-OWN [Sonos*] OWNED [DirtyLittleBirdyFeet*]

## A Flexible Network Adapter Is Key to Wi-Fi Hacking

A powerful wireless network adapter with the ability to inject packets and listen in on Wi-Fi conversations around it gives any hacker an advantage over the airwaves. It can be confusing picking the right adapter for you, but by carefully checking the chipset contained, you can ensure you won't be surprised when you make your purchase. If you already have an adapter, putting it through its paces before using it in the field is recommended before you rely on it for anything too important.

I hope you enjoyed this guide to testing your wireless network cards for packet injection and wireless monitor mode. If you have any questions about this tutorial on Kali-compatible wireless network adapters or you have a comment, feel free to reach me on Twitter

Chipsets that work with Kali include:

- Atheros AR9271

- Ralink RT3070

- Ralink RT3572

- Realtek 8187L (Wireless G adapters)

- Realtek RTL8812AU (newly in 2017)

- my research also suggests the Ralink RT5370N is compatible

In 2017, Kali Linux began supporting drivers for the RTL8812AU wireless chipsets. These drivers are not part of the standard Linux kernel and have been modified to allow for injection. This is a big deal because this chipset is one of the first to support 802.11 AC, bringing injection-related wireless attacks to this standard.

Most Popular Kali Linux Pen Testing Wireless Chipsets.
Atheros AR9271
Ralink RT3070
Ralink RT3572
Ralink RT5572
Realtek RTL8812AU
Ralink RT5370N

| ADAPTER NAME | CHIPSET | FREQUENCY | PROTOCOL | ANTENNA |
| --- | --- | --- | --- | --- |
| Alfa AWUS036NHA | Atheros AR9271 | 2.4GHz | 802.11N | External |
| Alfa AWUS036NH | Ralink RT3070 | 2.4GHz | 802.11N | External |
| Alfa AWUS036ACH | RTL8812AU | 2.4GHz/5Ghz | 802.11AC | External |
| TP-Link TL-WN722N V1 | Atheros AR9271 | 2.4GHz | 802.11N | External |
| Panda PAU09 | Ralink RT5572 | 2.4GHz | 802.11N | External |
| Panda PAU05 | Ralink RT5572 | 2.4GHz | 802.11N | Internal |
| Alfa AWUS036NEH | Ralink RT3070 | 2.4GHz | 802.11N | External |
| Alfa AWUS051NH | Ralink RT3572 | 2.4GHz/5Ghz | 802.11N | External |
| Alfa AWUS036H | Realtek 8187L | 2.4GHz | 802.11G | External |

## What is the best wireless card to buy ?

Which card to purchase is a hard question to answer. Each person's criteria is somewhat different, such as one may require 802.11ax capability, or may require it to work via virtualization. However, having said that, then the following cards are considered the best in class:

- Alfa AWUS036ACH (a/b/g/n/ac) is the best performing card, but the driver can be unstable enough to crash your kernel

- Alfa AWUS036ACM (a/b/g/n/ac) is the highest performing of the STABLE devices, but it requires kernel 4.19.5 or higher, and the driver doesn't work on the Raspberry Pi 3 yet; it works on the Raspberry Pi 4.

Runner ups:

- Alfa AWUS036H [b/g USB]

- Ubiquiti SRC [a/b/g Cardbus]

- Ubiquiti SRX [a/b/g ExpressCard]

- Airpcap series [USB]

- TP-Link TL-WN722N v1 [b/g/n USB] - Beware, if version is not specified by vendor, it is NOT v1

- Alfa AWUS036NHA [b/g/n USB]

- Alfa AWUS051NH v2 [a/b/g/n USB]

- MiniPCIe: anything that uses ath9k, especially AR92xx and AR93xx (ability to do spectral scan)

Also read this first before purchasing. There are many available on the market for fairly low prices. You are simply trading off distance, sensitivity and performance for cost.

If you want to know if your existing card is compatible then use this page: Tutorial: Is My Wireless Card Compatible?